PROJECT LOG BOOK

Project Name: TinyML-Based Vibration Anomaly Detection System for Predictive Maintenance

| DATE | WORK DONE | SIGNATURE |
| --- | --- | --- |
| 5-08-2026 | Pitch our idea in front of our guide and project coordinator , explain the previous works related to project with help of research papers taken some suggestion ,finalize the project with rough idea of the hardware requirement |  |
| 6-08-2026 | Our team member Parth Ghare visited his internship company “Avyanna Tech pvt limited” for purpose of sponsorship and talk about the project to his mentor for further discussions |  |
| 7-08-2026 | Finalize the main hardware of our project-the single phase induction motor and esp-32 dev kit for MCU and mpu6050 as a sensor for vibration |  |
| 8-08-2026 | Created the repositories and folder where our files will be saved in the software side ,created empty csv file for further appending of data and search for the motor/pump dataset in Kaggle |  |
| 9-08-2026 | Configured software environment for ESP32 data acquisition PARAMETERS: Arduino IDE;ESP32 Arduino core ; Adafruit MPU6050;PySerial Software environment configured Hardware not yet available; physical sensor verification pending |  |
| 10-08-2026 | Downloaded and explored the CWRU bearing dataset. Organized the selected Healthy, Ball, Inner Race and Outer Race MATLAB files in the project raw-data folder and inspected the available DE vibration signals and RPM information. |  |
| 11-08-2026 | Calculated CWRU bearing characteristic frequencies using the corrected bearing parameters and RPM-dependent formulas. Performed FFT analysis and saved frequency-domain plots for checking 1×, 2×, 3× and bearing-frequency components. |  |
| 12-08-2026 | Extracted vibration features including RMS, variance, kurtosis, crest factor, peak-to-peak value and FFT/physics-based energy features. Checked the class distribution and generated feature boxplots for Healthy, Ball, Inner Race and Outer Race classes. |  |
| 13-08-2026 | Created a new feature-analysis notebook. Performed feature correlation, statistical analysis and feature-distribution analysis. Prepared an exploratory Random Forest baseline using RMS, variance, kurtosis, crest factor and peak-to-peak features and inspected feature importance. |  |
| 14-08-2026 | Created the window-dataset preparation workflow. Defined 1-second windows at 12 kHz with 50% overlap and prepared recording-level grouping so windows from the same CWRU recording can be kept together during later dataset splitting. |  |
| 16-08-2026 | Created a new dataset-split and window-generation notebook. Performed recording-level train/validation/test grouping, checked for data leakage, generated 1-second 12000-sample vibration windows and saved the resulting metadata and window files for model development. |  |